TECHNICAL SPECIFICATION

for a public procurement with subject matter:

“Replacement of the exterior windows and doors of an apartment building, forming part of the immovable properties, owned by the Republic of Bulgaria outside the country, provided for administration to the MFA, with address: 328 East 86th Street, New York, NY 10028, USA

Project: “Apartment building, owned by the Bulgarian state, with address: 328 East 86th Street, New York, NY 10028, USA”

Contracting Authority: The Permanent Representation of the Republic of Bulgaria in New York

1.  Subject matter of the terms of reference for the tenders:

Replacement of all exterior windows and doors in an edifice in accordance with attached list and schemes of doors and windows, of an approximate area of 380 sq. m (4,090 sq.ft.) and restoration of the ledges, damaged in the process of installation of the old frames (facing and openings), the window sills and reveals. The precise dimensions of each of the elements will be specified in the process of taking of the actual measurements on site. The indicative dimensions, the manner of opening and the window and door types are shown in the scheme attached - for each of them individually. The frames articulation, the manner of opening and the color (dark brown - to match the existing frames), will remain unchanged, in order to preserve the overall appearance of the facade. The tenders may concern aluminum frames with interrupted thermal bridge or without interrupted thermal bridge.

2.  Required certificates of quality and compliance of the input materials

- The companies must attach to their tenders quality certificates and declarations of conformity of all input materials and elements, including hardware, mechanisms, seals, brushes, glazed units, fixing devices, materials for finishing works, in accordance with the applicable normative framework and the requirements in the respective administrative district.

- The tenders must certainly be accompanied also by certificates of original type testing of the products, conducted by the respective licensed laboratories, as well as of sound insulation, heat insulation, fire resistance, tightness to water, in accordance with the standards of the applicable normative framework of the respective administrative district.

3. Technical requirements as to the windows and doors and the elements input into them, as well as to the materials for performance of finishing works.

The frame profile must conform to the current energy efficiency requirements, in accordance with the system used, in the following alternatives:

- Aluminum profiles with interrupted thermal bridge or without interrupted thermal bridge depending on the system used and the dimensions of glazed units (double pane or triple pane - four season).

Glazed unit - double pane or triple pane, in conformity with the current technical requirements and of a high level of noise insulation and heat insulation.

The glass input into the glazed unit, must be of the low-emissivity K glass type - the so-called four season combination.

The glazed unit thickness shall be 40 mm.

The installation shall be performed according to details and instructions of the manufacturer, in order to ensure the reliability and integrity of the chambers.

The fixing of the frames to the building walls must ensure reliability, taking into account the operating requirements and atmospheric - corrosion-resistant fixing elements must be used.

After the installation of the frames, a high quality protection must be ensured for the joint between the frames and the bordering structure, using expanding foam and self-adhesive insulating according to the manufacturer’s instructions.

The sealings must be weather-resistant. Only EPDM, silicone or equivalent sealings are allowed.

The brushes must be highly resistant to wear.

The openings necessary for draining away the moisture of condensation and rainwater must be envisaged in the frame profile. Clogging up and damage to the respective openings must be prevented in the course of installation of the window sills.

The hardware of the frames must be reliable and ensure fast shutting (usage of security hardware recommended).

It must be envisaged to restore, after the installation of the frames, any ledges damaged, namely by facing by gypsum plastering or drywall in the interior, restoration of the outer side of the facade around the windows using the respective material (frost and humidity resistant) and replacement of the window sills and ledges.

The coefficient of heat passage (U-value) must be in compliance with current energy efficiency requirements in conformity with the system proposed.

4. Requirements in regard to the participants in the public procurement.

The companies shall attach to their tenders documents, certifying that as at the moment of participation in the public procurement they are licensed to perform the activities, subject of these terms of reference, in compliance with the applicable legislation and the requirements of the local administration.

The companies shall ensure all required permits, ancillary to the performance of the procurement, required by the local administration in relation to repair activities, installation and dismantling of the windows and doors, rendering safe and marking of the surrounding area in the course of performing the respective activities, the removal and disposal of the old windows and doors and construction waste.

- type of certificates of quality and compliance of all input materials and elements, in accordance with the current normative framework and the requirements of the respective administrative district;

Certificates issued by National Certified Testing Laboratories for the products, which must indicate equivalent or higher performance indicators than the minimum ones required under the following standards

Standard/Specification for Windows, Doors and Skylights, AAMA/WDMA/CSA 101/I.S.2/A440

or

ASTM Standards

- kind of the certificates of original type testing of the products, conducted by the respective licensed laboratories, as well as of sound insulation, heat insulation, fire resistance, tightness to water, in accordance with the standards of the applicable normative framework of the respective administrative district.

Certificates issued by National Certified Testing Laboratories for the products, which must indicate equivalent or higher performance indicators than the minimum ones required under the following standards

Standard/Specification for Windows, Doors and Skylights, AAMA/WDMA/CSA 101/I.S.2/A440

or

ASTM Standards

- the requisite permits, ancillary to the performance of the procurement, required by the local administration in relation to repair activities, installation and dismantling of the joinery, rendering safe and marking of the surrounding area in the course of performing the respective activities, the removal and disposal of the old joinery and construction waste;

- Holding of license to perform construction activities, issued by the New York City Department of Buildings of the government of New York City.

- Insurance - requirements.

The Licensed Tenderer, registered with the Department of Buildings, must hold insurance policies that are in effect and valid.

The Licensed Tenderer must fulfil the following insurance requirements:

General third-party liability insurance Must be of at least one million US Dollars per insured event.

Workers’ compensation insurance: In order to be eligible to obtain renewal of permit, the Tenderer shall ensure and maintain workers’ compensation insurance, as required by the law.

Disability insurance: as a condition for issuance and renewal of a permit the Tenderer shall ensure and maintain insurance for payments in case of disability, as required by the law.

- Permit from the government of New York City to implement activities for rendering the works safe - issued in regard to the principal contractor or a subcontractor.

- Certificate for installation of windows and doors – InstallationMasters™, other certificates, issued by licensed companies in regard to the principal contractor or a subcontractor.

The companies must include in the value of their tenders all waste removal and disposal charges and the fees for all permits required.

The tenderers shall attach a detailed list of the types and quality of the materials, which they intend to input in the course of performance of the activities. They must also describe all certificates of product compliance and permits for the input of various products, by indicating the manufacturers and attaching to their technical proposal certified copies of the documents, proving the compliance of the products and their fitness for use in accordance with their purpose.

The companies must harmonize all activities performed with the requirements for health and safety at work.

The Permanent Representation of the Republic of Bulgaria in New York is exempted from payment of taxes and charges. The tenders and the payments in regard to them shall be net of taxes and the Permanent Representation shall issue the requisite documents to the companies for submission to the revenue bodies.

5. Procurement performance term

The replacement of the windows and doors must be completed in 90 (ninety) calendar days, as of the signing of the contract, under the following conditions:

- Envisaging of a linear schedule for phased replacement (by floors) of the windows and doors, so as to allow for the installation to commence not later than 30 (thirty) days as of the signing of the contract.

- After dismantling the old ones and installing the new windows and doors, the restoration of any ledges damaged by the repair activities around the windows of the respective floor (apartment) must commence within 24 hours from the installation.

6. Warranties

The tenderers’ offers must envisage the following minimum warranty terms for delivery and installation of the windows and doors:

- a five-year warranty for aluminum or PVC profiles and for sealings.

- a five-year warranty for glazed units - in case of leakage.

- a five-year warranty for functionality of the hardware and accessories.

- a five-year warranty for the quality of installation of the windows and doors and the finishing works performed.

7. Tender validity term - 90 (ninety) calendar days.